#CODE:     (CLASS PROGRAM )

// CONFIG

#pragma config FOSC = EXTRC     // Oscillator Selection bits (RC oscillator)

#pragma config WDTE = OFF       // Watchdog Timer Enable bit (WDT disabled)

#pragma config PWRTE = OFF      // Power-up Timer Enable bit (PWRT disabled)

#pragma config BOREN = OFF      // Brown-out Reset Enable bit (BOR disabled)

#pragma config LVP = OFF        // Low-Voltage (Single-Supply) In-Circuit Serial Programming Enable bit (RB3 is digital I/O, HV on MCLR must be used for programming)

#pragma config CPD = OFF        // Data EEPROM Memory Code Protection bit (Data EEPROM code protection off)

#pragma config WRT = OFF        // Flash Program Memory Write Enable bits (Write protection off; all program memory may be written to by EECON control)

#pragma config CP = OFF         // Flash Program Memory Code Protection bit (Code protection off)

#include<xc.h>

#define _XTAL_FREQ 6000000

// Declare PWM duty cycle high and low byte variables

unsigned char pwm10_high, pwm10_low;

unsigned char pwm50_high, pwm50_low;

unsigned char pwm80_high, pwm80_low;

// Function prototypes

void pwm_init(void);

void pwm_dutycycle_update(void);

void main()

{

pwm_init();  // Initialize PWM settings

while (1)

{

pwm_dutycycle_update();  // Update PWM duty cycle continuously

}

}

// Define PWM duty cycle high and low byte values

void pwm_init(void)

{

TRISC = 0xFB;    // Configure RC2 as output (1111 1011)

CCP1CON = 0x0C;  // Set PWM mode for CCP1

T2CON = 0x06;    // Enable Timer2 with prescaler 1:16

PR2 = 0x5E;      // Set PWM period

// Initialize PWM duty cycle values

// Additional configurations can be added based on requirements

pwm10_high = 0x09;

pwm10_low  = 0x02;

pwm50_high = 0x2F;

pwm50_low  = 0x00;

pwm80_high = 0x4B;

pwm80_low  = 0x00;

}

// Function to update PWM duty cycle

void pwm_dutycycle_update(void)

{

// Set 10% duty cycle

CCPR1L = pwm10_high;  // High byte (0x09)

CCP1CON = (CCP1CON & 0xCF) | (pwm10_low << 4); // Low byte (0x02)

__delay_ms(3000);  // Delay for 3 seconds

// Set 50% duty cycle

CCPR1L = pwm50_high;  // High byte (0x2F)

CCP1CON = (CCP1CON & 0xCF) | (pwm50_low << 4); // Low byte (0x00)

__delay_ms(3000);  // Delay for 3 seconds

// Set 80% duty cycle

CCPR1L = pwm80_high;  // High byte (0x4B)

CCP1CON = (CCP1CON & 0xCF) | (pwm80_low << 4); // Low byte (0x00)

__delay_ms(3000);  // Delay for 3 seconds

}

CLASS5:  (TASK PROGRAM)

// CONFIG

#pragma config FOSC = EXTRC     // Oscillator Selection bits (RC oscillator)

#pragma config WDTE = OFF       // Watchdog Timer Enable bit (WDT disabled)

#pragma config PWRTE = OFF      // Power-up Timer Enable bit (PWRT disabled)

#pragma config BOREN = OFF      // Brown-out Reset Enable bit (BOR disabled)

#pragma config LVP = OFF        // Low-Voltage (Single-Supply) In-Circuit Serial Programming Enable bit (RB3 is digital I/O, HV on MCLR must be used for programming)

#pragma config CPD = OFF        // Data EEPROM Memory Code Protection bit (Data EEPROM code protection off)

#pragma config WRT = OFF        // Flash Program Memory Write Enable bits (Write protection off; all program memory may be written to by EECON control)

#pragma config CP = OFF         // Flash Program Memory Code Protection bit (Code protection off)

#include<xc.h>

#define _XTAL_FREQ 6000000

// Declare PWM duty cycle high and low byte variables

unsigned char pwm20_high, pwm20_low;

unsigned char pwm60_high, pwm60_low;

unsigned char pwm90_high, pwm90_low;

// Function prototypes

void pwm_init(void);

void pwm_dutycycle_update(void);

void main()

{

pwm_init();  // Initialize PWM

while (1)

{

pwm_dutycycle_update();  // Update PWM duty cycle continuously

}

}

// Define PWM duty cycle high and low byte values

void pwm_init(void)

{

TRISC = 0xFB;    // Configure RC2 as output (1111 1011)

CCP1CON = 0x0C;  // Set PWM mode for CCP1

T2CON = 0x06;    // Enable Timer2 with prescaler 1:16

PR2 = 0x2E;      // Set PWM period

// Initialize PWM duty cycle values

pwm20_high = 0x09;   //0000 1001

pwm20_low  = 0x02;   //0000 0010

pwm60_high = 0x1C;    //0001 1100

pwm60_low  = 0x01;    //0000 0001

pwm90_high = 0x2A;    //0010 1010

pwm90_low  = 0x01;    //0000 0001

}

// Function to update PWM duty cycle

void pwm_dutycycle_update(void)

{

// Set 20% duty cycle

CCPR1L = pwm20_high;  // High byte (0x09)

CCP1CON = (CCP1CON & 0xCF) | (pwm20_low << 4); // Low byte (0x02)

__delay_ms(3000);  // Delay for 3 seconds

// Set 60% duty cycle

CCPR1L = pwm60_high;  // High byte (0x1C)

CCP1CON = (CCP1CON & 0xCF) | (pwm60_low << 4); // Low byte (0x01)

__delay_ms(3000);  // Delay for 3 seconds

// Set 90% duty cycle

CCPR1L = pwm90_high;  // High byte (0x2A)

CCP1CON = (CCP1CON & 0xCF) | (pwm90_low << 4); // Low byte (0x01)

__delay_ms(3000);  // Delay for 3 seconds

}